PM-EB2NLFR-W – A&E SPECIFICATION SHEET

DIVISION 28 – ELECTRONIC SAFETY AND SECURITY

28 20 00 – Electronic Surveillance
28 23 00 – Video Surveillance
28 23 29 – Video Surveillance Remote Devices and Sensors

This specification is intended for use by consultants, engineers, and system integrators preparing compliance documents for security system tenders.

PART 1 – GENERAL

SUMMARY

A. The camera shall be a 2MP Fixed Bullet Network Camera suitable for continuous indoor and outdoor surveillance.
B. The camera shall feature Color Night Vision, Digital WDR, and smart white-light activation based on event detection.
C. The camera shall be IP67-rated for weather resistance.
D. The camera shall comply with NDAA requirements.
E. The camera shall support H.265+/H.265/H.264+/H.264 compression.

WARRANTY

Provide the manufacturer’s standard  surveillance product warranty.

PART 2 – PRODUCTS

2MP Fixed Bullet Network Camera

GENERAL CHARACTERISTICS

The Fixed Bullet Camera shall:

- Deliver a maximum resolution of 2MP (1920×1080).

- Use a 1/3" CMOS image sensor.

- Provide 0.0462 lux color sensitivity at F2.2, AGC ON.

- Provide IR illumination range of 10–20m.

- Provide white-light illumination range of 10–20m.

- Include Digital WDR, BLC, HLC, Defog, 3DNR.

- Provide 2 simultaneous video streams.

- Include advanced cybersecurity: video encryption, firmware encryption, illegal login lock, HTTPS, IP filtering, 802.1x.

- Support ONVIF Profiles S & T.

CAMERA

| Specification | Details |
| --- | --- |
| Camera Type | 2MP Fixed Bullet Network Camera |
| Image Sensor | 1/3" CMOS |
| Max Resolution | 1920×1080 |
| Electronic Shutter | 1/3s – 1/100000s |
| Minimum Illumination | Color: 0.0462 lux @F2.2, AGC ON |
| IR Range | 10–20m |
| White Light Range | 10–20m |
| Day/Night | ICR (IR cut filter removable) |

LENS

| Specification | Details |
| --- | --- |
| Focal Length | 3.6mm @F2.2 |
| Focus | Fixed |
| Iris Type | Fixed Iris |
| Lens Mount | M12 |
| Horizontal FOV | 74° |
| Vertical FOV | 40° |
| Diagonal FOV | 85° |
| Angle Adjustment | Pan 0°–360° • Tilt 0°–80° • Rotation 0°–360° |

VIDEO

- Compression:

  - Main stream: H.265+/H.265/H.264+/H.264

  - Sub stream: same formats

- Supported Resolutions:

  - 1080P (1920×1080)

  - 720P (1280×720)

  - D1, VGA, 640×360, 480×240, CIF

- Frame Rates:

  - Main stream

    - 60Hz: 1–30fps @1080P/720P

    - 50Hz: 1–25fps @1080P/720P

  - Sub stream

    - 60Hz: 1–30fps @D1/VGA/640×360/480×240/CIF

    - 50Hz: 1–25fps |

- Bitrate: 64 Kbps – 3 Mbps (VBR/CBR)

- Image Controls: ROI, saturation, brightness, hue, contrast, digital WDR, sharpness, mirror, flip, noise reduction.

AUDIO

| Item | Specification |
| --- | --- |
| Audio Compression | G.711 A/U |
| Microphone | Built-in |

INTELLIGENT ANALYTICS

Smart Detection

- Line Crossing Detection

- Region Intrusion Detection

- Video Exception:

  - Scene change

  - Video blur

  - Abnormal colour detection

Motion Detection

- Supported

- SMD

DORI PERFORMANCE (EN 62676-4)

(Lens: 3.6mm)

| Function | Distance |
| --- | --- |
| Detect | 59m (194 ft) |
| Observe | 24m (79 ft) |
| Recognize | 12m (39 ft) |
| Identify | 6m (20 ft) |

NETWORK

| Item | Specification |
| --- | --- |
| Interface | RJ45 10/100M Ethernet |
| Protocols | UDP, IPv4, IPv6, DHCP, DDNS, NTP, SNMP, RTSP, 802.1x, UPnP, SMTP, FTP, HTTP/HTTPS, QoS |
| Browser Support | Chrome 89+, Edge 89+, Firefox 87+, Safari 14+ |
| Remote Monitoring | Web Browser, NVR, VMS , Mobile App |
| Concurrent Connections | Up to 3 users |
| ONVIF Compatibility | Profiles S & T |

INTERFACES

| Item | Details |
| --- | --- |
| Audio | 1 × Built-in MIC |

FUNCTIONS

- Watermark

- Video mask

- Heartbeat

- Cybersecurity Suite:

  - Video encryption

  - Firmware encryption

  - Configuration encryption

  - Illegal login lock

  - IP filtering

  - 802.1x authentication

  - Basic/digest authentication for HTTP/RTSP

ENVIRONMENTAL & POWER

| Item | Specification |
| --- | --- |
| Power Supply | PoE (IEEE 802.3af) |
| Power Consumption | < 5W |
| Weather Rating | IP67 |
| Operating Temperature | –20°C to +55°C |
| Storage Temperature | –20°C to +60°C |
| Humidity | <95% (non-condensing) |

MECHANICAL

| Item | Specification |
| --- | --- |
| Dimensions | 162.1 × 74.8 × 73.8 mm |
| Weight | Approx. 0.24 kg |
| Installation | Ceiling or Wall Mount |

CERTIFICATIONS

- EMC: EN 55032:2015+A11:2020, EN 55035:2017+A11:2020

- Safety: EN IEC 62368-1:2020+A11:2020

- RoHS: 2011/65/EU