PM-PB8ULVR-W – A&E SPECIFICATION SHEET

DIVISION 28 – ELECTRONIC SAFETY AND SECURITY

28 20 00 – Electronic Surveillance
28 23 00 – Video Surveillance
28 23 29 – Video Surveillance Remote Devices and Sensors

This specification is designed for tender submissions, consultant documentation, project BOQs, and compliance evaluations.

PART 1 – GENERAL

SUMMARY

A. The camera shall be an 8MP Varifocal Bullet Network Camera with 4.3× motorized optical zoom for medium- to long-range surveillance.
B. The device shall include 120 dB True Wide Dynamic Range (WDR) for high-contrast environments.
C. The camera shall provide Color Night Vision down to 0.015 lux @F1.4, AGC ON.
D. The system shall include IR illumination up to 50–70 meters.
E. The device shall support intelligent analytics such as line crossing, region intrusion, illegal parking, abandoned/missing object detection, and heat maps.
F. The camera shall be IP67 weatherproof, NDAA compliant, and support ONVIF Profiles S/G/M/T.

WARRANTY

Provide manufacturer’s standard  warranty for surveillance products.

PART 2 – PRODUCTS

8MP Varifocal Bullet IP Camera with 4.3× Motorized Zoom

GENERAL CHARACTERISTICS

The camera shall:

- Provide 8MP UHD resolution (3840×2160).

- Use a 1/2.8” CMOS sensor for enhanced low-light performance.

- Support 50–70m IR illumination for night surveillance.

- Include 2.8–12mm motorized varifocal lens with DC Auto Iris.

- Provide 3 simultaneous video streams.

- Offer video enhancements: WDR, BLC, HLC, DNR, ROI, sharpness, brightness, contrast, mirror, flip.

- Include audio input, audio output, and built-in MIC.

- Provide 1 alarm input / 1 alarm output.

- Support microSD storage up to 256GB.

CAMERA

| Item | Specification |
| --- | --- |
| Type | 8MP Varifocal Bullet Camera |
| Image Sensor | 1/2.8" CMOS |
| Max Resolution | 8MP (3840×2160) |
| Electronic Shutter | 1/3s – 1/100000s |
| Minimum Illumination | Color: 0.015 lux @F1.4, AGC ON |
| IR Range | 50–70m |
| Day/Night | ICR |

LENS

| Item | Specification |
| --- | --- |
| Focal Length | 2.8–12mm @F1.4 |
| Optical Zoom | 4.3× Motorized |
| Focus | Motorized |
| Iris Type | DC Auto |
| Lens Mount | Ø14 |
| Horizontal FOV | 102° – 32° |
| Vertical FOV | 54° – 17° |
| Diagonal FOV | 120° – 37° |

VIDEO

Compression

- Main stream: H.265+/H.265/H.264+/H.264

- Sub/Third stream: H.265+/H.265/H.264+/H.264/MJPEG

Supported Resolutions

- 8MP (3840×2160)

- 6MP (3072×2048)

- 5MP (2592×1944)

- 4MP (2592×1520)

- 2K (2560×1440)

- 3MP, 1080P, 720P, D1, 640×480, 480×240, CIF

Frame Rates

- Main Stream:

  - 60Hz → 1–30fps

  - 50Hz → 1–25fps

- Sub Stream:

  - 60Hz → 1–30fps

  - 50Hz → 1–25fps

- Third Stream:

  - 60Hz → 1–30fps

  - 50Hz → 1–25fps

Bitrate

64 Kbps – 12 Mbps (VBR/CBR)

Image Settings

ROI, saturation, brightness, hue, contrast, WDR, sharpness, mirror, flip, DNR, etc.

AUDIO

| Item | Specification |
| --- | --- |
| Audio Compression | G.711 A/U |
| Audio Input | 1 channel |
| Audio Output | 1 channel |
| Built-in MIC | Yes |

INTELLIGENT ANALYTICS

AI Analytics (Advanced)

- Face Detection

- Face Recognition & Matching (via NVR)

- Video Metadata:

  - Captures human body, motor vehicle, non-motor vehicle

  - 13 human attributes, 4 vehicle attributes, 1 non-motor vehicle attribute

- Audio Exception Detection

- People/vehicle statistics (NVR or SD card)

Smart Event Detection

- Line Crossing

- Region Intrusion

- Entrance/Exit detection

- Abandoned/Missing Object

- Loitering

- Illegal Parking

- Target Counting (line & area)

- Heat Map

Smart Alarms

- Motion detection

- SMD

- SD card full/error

- IP conflict

- Cable disconnection

- Video exception: scene change, blur, abnormal color

DORI PERFORMANCE

(Using 2.8–12mm motorized lens)

| Function | Wide | Tele |
| --- | --- | --- |
| Detect | 85m | 289m |
| Observe | 34m | 116m |
| Recognize | 17m | 58m |
| Identify | 8m | 29m |

INTERFACES

| Item | Specification |
| --- | --- |
| Network | RJ45 (10/100M) |
| Video Output | CVBS (BNC ×1) |
| Audio | 1 In / 1 Out + Built-in MIC |
| Storage | microSD slot, up to 256GB |
| Alarm I/O | 1 In / 1 Out |
| RS485 | Not supported |

NETWORK

- Protocols: UDP, IPv4/IPv6, DHCP, NTP, RTSP, RTP, RTCP, PPPoE, DDNS, SMTP, FTP, 802.1x, SNMP, UPnP, RTMP, HTTP/HTTPS, HTTP POST, P2P, QoS

- ONVIF: Profiles S/G/M/T

- Remote Monitoring: Web browser, NVR,  VMS , mobile app

- Browser Support: Chrome 89+, Edge 89+, Firefox 87+, Safari 14+

- Online Users: Up to 10 simultaneous users

FUNCTIONS

- Watermark

- IP Filtering

- Video Mask

- Heartbeat

- ANR

- Cloud Upgrade

- Corridor Mode

- Multi-stream real-time transmission

Cybersecurity

- Video encryption

- Firmware encryption

- Configuration encryption

- Illegal login lock

- IP filtering

- 802.1x authentication

- HTTP/RTSP basic & digest authentication

- NDAA compliant

ENVIRONMENTAL & POWER

| Item | Specification |
| --- | --- |
| Power Supply | DC12V / PoE |
| PoE Standard | IEEE 802.3af/at |
| Power Consumption | <12W |
| Weather Protection | IP67 |
| Operating Temp | –30°C to +60°C |
| Storage Temp | –30°C to +65°C |
| Humidity | <95% non-condensing |

MECHANICAL

| Item | Specification |
| --- | --- |
| Dimensions | 272.3 × 94.9 × 88.9 mm |
| Net Weight | Approx. 1.04 kg |
| Installation | Ceiling mount / Wall mount |

CERTIFICATIONS

- EMC: EN 55032:2015+A11:2020, EN 55035:2017+A11:2020

- Safety: EN IEC 62368-1:2020+A11:2020

- RoHS: 2011/65/EU