PM-VB4ULFT-W – A&E SPECIFICATION SHEET

DIVISION 28 – ELECTRONIC SAFETY AND SECURITY

28 20 00 – Electronic Surveillance
28 23 00 – Video Surveillance
28 23 29 – Video Surveillance Remote Devices and Sensors

This specification is suitable for engineering consultants, tender submissions, and project technical compliance documents.

PART 1 – GENERAL

SUMMARY

A. The camera shall be a 4MP Fixed Bullet True Color Network Camera delivering exceptional color imaging even in near-total darkness.
B. The imaging system shall be powered by AI-ISP, an advanced Image Signal Processor for real-time optimisation.
C. The camera shall support full-color night vision down to 0.0001 lux @F1.0, enabled by a stacked large-format CMOS sensor and ultra-large aperture F1.0 lens.
D. The camera shall support 120 dB wide dynamic range, advanced encryption, secure access control, and NDAA compliance.
E. The camera shall provide 20–30 m white-light illumination for night surveillance.
F. The housing shall be IP67 for outdoor installation.

WARRANTY

Provide manufacturer’s standard  warranty.

PART 2 – PRODUCTS

4MP Fixed Bullet True Color IP Camera

GENERAL CHARACTERISTICS

The camera shall:

- Capture 4MP (2688×1520) resolution video with color fidelity maintained in extremely low illumination.

- Provide 0.0001 lux color night performance @F1.0 (AGC ON).

- Use a 1/1.79″ CMOS sensor for maximum light intake and low-noise imaging.

- Offer digital 120 dB WDR, BLC, HLC, and 3D NR.

- Provide three simultaneous streams (H.265+/H.265/H.264+/H.264/MJPEG).

- Support up to 512GB edge storage via microSD card.

- Provide audio input + built-in microphone.

- Integrate ONVIF Profiles S/G/M/T.

- Support AI functions including face detection and audio exception detection.

CAMERA

| Item | Specification |
| --- | --- |
| Type | 4MP Fixed Bullet True Color Network Camera |
| Image Sensor | 1/1.79″ CMOS |
| Max Resolution | 4MP (2688×1520) |
| Electronic Shutter | 1/3s – 1/100000s |
| Minimum Illumination | Color: 0.0001 lux @F1.0 (AGC ON) |
| Illumination Range | 20–30 m white-light view distance |
| Day/Night | – (True Color camera, no IR mode) |

LENS

| Item | Specification |
| --- | --- |
| Focal Length | 2.8mm @F1.0 |
| Focus | Fixed |
| Iris | Fixed Iris |
| Lens Mount | M12 |
| Field of View | H: 112° • V: 61° • D: 132° |
| Angle Adjustment | Pan: 0–360° • Tilt: 0–90° • Rotate: 0–360° |

VIDEO

Compression Formats

- Main: H.265+/H.265/H.264+/H.264

- Sub & Third Stream: H.265+/H.265/H.264+/H.264/MJPEG

Supported Resolutions

- 4MP (2688×1520)

- 2K (2560×1440)

- 1080P, 720P

- D1, VGA, 640×360, 480×240, CIF

Frame Rates

- Main Stream:

  - 60Hz → 1–30fps

  - 50Hz → 1–25fps

- Sub / Third Stream:

  - 60Hz → 1–30fps

  - 50Hz → 1–25fps

Bitrate

- 32 Kbps – 8 Mbps, VBR / CBR

Image Settings

ROI, saturation, brightness, hue, contrast, WDR, sharpness, mirror, flip, NR — fully configurable.

AUDIO

| Item | Specification |
| --- | --- |
| Audio Compression | G.711 A/U |
| Audio Input | 1 channel |
| Built-in Microphone | Yes |

INTELLIGENT ANALYTICS

AI-Enhanced Analytics

- Face Detection

- Audio Exception Detection

- Face Recognition / Matching: Yes (via NVR)

Video Metadata

- Human, vehicle, two-wheelers attribute metadata supported.

Smart Events

- Line crossing

- Region intrusion

- Entrance/exit detection (human/vehicle/non-vehicle classification)

- Abandoned/missing object detection

- Target counting by line

Smart Alarm

- Motion detection

- SMD

- SD card full/error

- IP address conflict

- Network disconnection

- Scene change / blur / abnormal image detection

DORI DISTANCE (EN 62676-4)

2.8mm fixed lens

| Function | Distance |
| --- | --- |
| Detect | 57m (187 ft) |
| Observe | 23m (75 ft) |
| Recognize | 11m (36 ft) |
| Identify | 6m (20 ft) |

INTERFACES

| Item | Specification |
| --- | --- |
| Network | RJ45 (10M/100M Ethernet) |
| Audio | 1ch audio input + built-in MIC |
| Storage | microSD slot, supports up to 512GB |
| RS485 | Not supported |

NETWORK

- Protocols: UDP, IPv4, IPv6, DHCP, NTP, RTSP, RTP, RTCP, RTMP, FTP, PPPoE, DDNS, SNMP, HTTP, HTTPS, 802.1x, UPnP, HTTP POST, P2P, QoS

- Interface Protocol: ONVIF Profiles S/G/M/T

- Browser Support: Chrome 89+, Edge 89+, Firefox 87+, Safari 14+

- Remote Monitoring: Web browser, NVR,  VMS, Mobile App

- Max Users: Up to 10 simultaneous users

FUNCTIONS

- Watermark

- Video Mask

- Heartbeat

- Image distortion correction

- Corridor mode

- ANR

- Cloud upgrade

- Disarming

Cybersecurity

- Video / firmware / configuration encryption

- Illegal login lock

- IP filtering

- 802.1x authentication

- Basic & Digest auth for HTTP/RTSP

- NDAA compliant

ENVIRONMENTAL & POWER

| Item | Specification |
| --- | --- |
| Power Supply | DC12V / PoE (IEEE 802.3af) |
| Power Consumption | < 6.5W |
| Weather Rating | IP67 |
| Operating Temp | –30°C to +60°C |
| Humidity | <95% (non-condensing) |
| Storage Temp | –30°C to +65°C |

MECHANICAL

| Item | Specification |
| --- | --- |
| Dimensions | 218.1 × 72.9 × 73.1 mm |
| Weight | Approx. 0.64 kg |
| Installation | Ceiling or wall mount |
| Appearance | Page 4 shows side & front elevations with labelled dimensions |

CERTIFICATIONS

- EMC: EN 55032:2015+A11:2020, EN 55035:2017+A11:2020

- Safety: EN IEC 62368-1:2020+A11:2020

- RoHS: 2011/65/EU