PM-VB8ULVC-W – A&E SPECIFICATION SHEET

DIVISION 28 – ELECTRONIC SAFETY AND SECURITY

28 20 00 – Electronic Surveillance
28 23 00 – Video Surveillance
28 23 29 – Video Surveillance Remote Devices and Sensors

This specification is intended for consultant approvals, tender submissions, technical design, and compliance documentation.

PART 1 – GENERAL

SUMMARY

A. The camera shall be an 8MP Varifocal Full-Color Bullet Network Camera with 4.3× motorized optical zoom for indoor and outdoor surveillance applications.
B. The imaging system shall provide 120 dB WDR, 0.02 lux color night vision, and dual illumination via IR and white LED light.
C. The camera shall support H.265+/H.265/H.264+/H.264/MJPEG encoding with three simultaneous streams.
D. Intelligent alarm functions shall include line crossing, region intrusion, abandoned/missing object detection, and smart alarms.
E. The housing shall be IP67-rated and comply fully with NDAA, offering advanced encryption and access control.

WARRANTY

standard warranty applies.

PART 2 – PRODUCTS

8MP Varifocal Bullet IP Camera (4.3× Motorized Zoom)

GENERAL CHARACTERISTICS

The camera shall:

- Deliver 8MP resolution (3840×2160).

- Use a 1/2.8″ CMOS sensor.

- Support color night vision: 0.02 lux @F1.6 (AGC ON).

- Provide IR illumination: 30–50 m and white-light illumination: 30–40 m.

- Support motorized varifocal 2.8–12 mm lens.

- Support 3 simultaneous video streams.

- Include 1 audio input and built-in microphone.

- Support microSD storage up to 256GB.

- Integrate ONVIF Profiles S/G/M/T.

CAMERA

| Item | Specification |
| --- | --- |
| Type | 8MP Varifocal Bullet Full-Color Camera |
| Image Sensor | 1/2.8″ CMOS |
| Max Resolution | 3840×2160 (8MP) |
| Electronic Shutter | 1/3s–1/100000s |
| Minimum Illumination | Color: 0.02 lux @F1.6, AGC ON |
| IR Range | 30–50 m |
| White-Light Range | 30–40 m |
| Day/Night | ICR |

LENS

| Item | Specification |
| --- | --- |
| Focal Length | 2.8–12 mm @F1.6 |
| Optical Zoom | 4.3× motorized |
| Iris | Fixed |
| Focus | Motorized |
| Lens Mount | Ø14 |
| Field of View | H: 97.3°–34° • V: 52.3°–17.7° • D: 116°–37° |
| Adjustment | Pan: 0–360° • Tilt: 0–90° • Rotate: 0–360° |

VIDEO

Compression

- Main stream: H.265+/H.265/H.264+/H.264

- Sub/Third streams: H.265+/H.265/H.264+/H.264/MJPEG

Supported Resolutions

- 8MP (3840×2160)

- 6MP (3200×1800)

- 4MP (2688×1520)

- 2K (2560×1440)

- 1080P / 720P

- D1, VGA, 640×360, 480×240, CIF

Frame Rates

- Main Stream:

  - 8MP: 1–20fps

  - 60Hz → 6MP/4MP/2K/1080P @ 1–30fps

  - 50Hz → 6MP/4MP/2K/1080P @ 1–25fps

- Sub Stream:

  - 60Hz: 720P/D1/VGA/640×360/CIF/480×240 (1–30fps)

  - 50Hz: same (1–25fps)

- Third Stream:

  - 60Hz: 720P/D1/640×360/CIF (1–30fps)

  - 50Hz: 1–25fps

Bitrate

- 64 Kbps – 12 Mbps

- VBR / CBR

Image Settings

ROI, saturation, brightness, hue, contrast, WDR, sharpness, mirror, flip, and NR.

AUDIO

| Item | Specification |
| --- | --- |
| Audio Compression | G711A/U |
| Audio Input | 1 channel |
| Built-in Microphone | Yes |
| Audio Output | Not supported |

INTELLIGENT ANALYTICS

Smart Alarm

- Motion detection

- SMD

- SD card full / error

- IP address conflict

- Cable disconnection

- Video exception (scene change, blur, abnormal colors)

Smart Event

- Line crossing

- Region intrusion (Human / Vehicle / Non-motor classification)

- Abandoned/missing object detection

DORI PERFORMANCE (EN 62676-4)

| Function | Wide (2.8 mm) | Tele (12 mm) |
| --- | --- | --- |
| Detect | 88 m | 277 m |
| Observe | 35 m | 111 m |
| Recognize | 17 m | 55 m |
| Identify | 8 m | 28 m |

INTERFACES

| Item | Specification |
| --- | --- |
| Network | RJ45 (10M/100M Ethernet) |
| Audio | 1 In, built-in MIC |
| Storage | microSD up to 256GB |

NETWORK

- Protocols: UDP, IPv4/IPv6, DHCP, NTP, RTSP, RTP, RTCP, PPPoE, DDNS, SMTP, FTP, 802.1x, SNMP, UPnP, RTMP, HTTP, HTTPS, HTTP POST, P2P, QoS

- ONVIF: Profiles S / G / M / T

- Supports multi-stream real-time transmission

- Simultaneous Users: Up to 10

- Browser Support: Chrome 89+, Edge 89+, Firefox 87+, Safari 14+

- Supports NVR, Web browser,  VMS , and Mobile App

FUNCTIONS

- Watermark

- IP address filtering

- Video mask

- Heartbeat

- Smart Supplement Light

- Corridor mode

- Image distortion correction

- ANR

- Cloud upgrade

Cybersecurity

- Video / firmware / configuration encryption

- Illegal login lock

- 802.1x authentication

- Basic & Digest HTTP/RTSP authentication

- NDAA compliant

ENVIRONMENTAL & POWER

| Item | Specification |
| --- | --- |
| Power Supply | DC12V / PoE (IEEE 802.3 af/at) |
| Power Consumption | < 12W |
| Weather Protection | IP67 |
| Operating Temperature | –30°C to +60°C |
| Storage Temperature | –30°C to +65°C |
| Humidity | <95% (non-condensing) |

MECHANICAL

| Item | Specification |
| --- | --- |
| Dimensions | 218.1 × 72.9 × 73.1 mm |
| Net Weight | Approx. 0.7 kg |
| Installation | Ceiling mount / Wall mount |

Appearance Diagram (Page 4)

- Side elevation showing 218.1 mm length, 70.4 mm height

- Front elevation width 79.5 mm

CERTIFICATIONS

- EMC: EN 55032:2015 + A11:2020; EN 55035:2017 + A11:2020

- Safety: EN IEC 62368-1:2020 + A11:2020

- RoHS: 2011/65/EU